HILEC 310B SERIES

VINYL COATED FIBERGLASS SLEEVING

| THERMAL CLASS: 130C | MIL-I-3190…MIL-I-21557 |
| --- | --- |
| # IEEE service temperature: -20C to 130C | FLAMMABILITY: VW-1 |
| # NEMA TF-1, TYPE 3 | UL FILE E86619 & E123956 |

CSA FILE 094072

| DESCRIPTION 310B SERIES is comprised of specially formulated poly vinyl chloride resin applied to premium electrical grade braided fiberglass sleeving. 310B SERIES is flame resistant, highly flexible, extremely abrasion resistant sleeving designed for use in class 130C insulating systems. 310B SERIES is compatible with most commonly used insulating resin systems. | #### RECOMMENDED USE 310B SERIES is used as supplementary insulation on motors, generators, and transformers. Additionally, 310B SERIES has found applications in the radio, television and electronics industries as terminal insulation where operating temperatures are 130C or lower. Its flame resistance renders 310B SERIES ideal for applications involving soldered splices and terminations. |
| --- | --- |

# PHYSICAL PROPERTIES (coating)

| ### Thermal endurance | 130C |  | ## Resistance to: Corona | Excellent |
| --- | --- | --- | --- | --- |
| ### Flame resistance | Self-extinguishing |  | ### Acids | Excellent |
| Cold brittle point | -20c maximum |  | Weak alkalis | Excellent |
| Oil resistance (24 hr @ 105C | No swelling or disintegration |  | Esters | Excellent |
|  |  |  | Fungus | Meets MIL-I- 631 |

| ##### DIELECTRIC GRADE | ##### PRODUCT NUMBER | MIN. AVE. BREAKDOWN | MIN. INDIV. |
| --- | --- | --- | --- |
| A | ##### 317B | 8000 | 6000 |
| B | 314B | 4000 | 2500 |
| C-1 | 312B | 2500 | 1500 |

#### AVAILABILITY

# SIZES: 310B SERIES is available in continuous spooled lengths or cut lengths in NEMA sizes #24 through 1”

- COLORS: 310B SERIES is available in black and white. Other colors may be available subject to minimum order quantities and/or setup charges.

- BRANDNAMES: 310 SERIES is also available as TURBOTUF™, VOL-FLEX™ or VG-41™

The above data is informational only and should not be used for specification purposes. 11/01